Programmable High-Power Optical Waveform Synthesis for Material Processing Using a Phase-Controlled Array of Spectrally Spaced Continuous-Wave Fiber Lasers

I am considering an array of 81 single-mode, 100 W continuous-wave (CW) fiber lasers, each operating at a precisely defined Bragg wavelength between 1040.222 nm and 1069.900 nm. Each laser has an electro-optically adjustable phase.

By appropriately controlling the relative phases and coherently combining the fiber outputs, the resulting waveform forms a pulse train with pulses separated by 3 mm in propagation distance. Each pulse has a temporal full width at half maximum (FWHM) of approximately 100 femtoseconds and a peak power of approximately 0.8 MW.

The phases of the individual lasers are adjusted sequentially by measuring the output pulse peak power and optimizing the phase of each channel.

Such a single-mode laser system could be used for high-precision material processing, combining high average power with ultrashort pulses.